Legal Description Abbreviations

Abbreviation	Definition	Use

AC	Acre

ADD	Addition	Usually refers to a subdivision.

ALG	Along	Used to describe a direction along a road, stream, etc…

APP	Approximately

ARC	Arc	The arc of a curve.

BEG	Beginning 	Beginning at a point…

CH	Chord	Direction of a curve along a straight line from point to

Point much like the string of a bow.

COM	Commencing	Starting point

COR	Corner

DEG	Degrees

DES	Described

E.	East	Compass Direction

ESMT	Easement	Protected area for utilities or access. Non-buildable

EXC.	Excluding	Usually used to identify a right-of-way or other

easement which is excluded from the property’s

described area

FT	Feet

LN	Line

N.	North	Compass Direction POB	Point of Beginning

NWLY	NorthWesterly	Compass Direction also NEly, SEly, SWly

PG	Page

POB	Point of Beginning

PT	Point	Starting Point

RGE or R.	Range	Same as Township. 36 Sq. mile land area.

Contains 36 sections but with a East or West

identification i.e. R 3W.

ROW	Right-of-way	Describes a roadway, or easement.

RR	Railroad

S.	South 	Compass Direction

SEC	Section 	1 sq. mile land area used in the PLSS System. Usually

followed by a no. i.e. Sec 23

SUB	Subdivision

TH	Thence	Start of a new line

Twn.or T	Township	36 Sq. mile land area. Contains 36 sections

followed by a no. and a North or South identification

i.e. T 3N

W.	West	Compass Direction

*or O	Degrees	Direction of measure, 360o range divided into 90o qtr

sections. These are Surveyor’s units ex. N.0o 15’ 47” E.

‘	Minutes	Smaller increment of measure. (see above example)

“	Seconds	Even smaller increment of measure (see above

example)

Ways that Legal Descriptions are written:

Public Land survey System- Section, Township and Range. Also known as the rectangular survey system.

EXAMPLE - PARCEL IDENTIFICATION NUMBER
This 16 digit number uniquely identifies every piece of property within the county for tax purposes.  The parcel id number is broken down as follows:

Example: 16-09-31-3-004-019.003

16: Locator - this number is derived from the township and range of the property.  The legend located on every tax map will depict this grid of locator numbers.

09: Area - this number refers the '4 section area' the property is located inside of.  See the legend on the map for details.

31: Section - this is the section number

3: Quarter-section - this identifies the quarter section (i.e. NE1/4=1, NW1/4=2, SW1/4=3, SE1/4=4)

004: Block - some maps are broken down into blocks because of the number of parcels per map, this number identifies which block on the map

019.003: Parcel and sub-parcel - this is the number found on the map.  Most often, leading zeros will not appear on the map. (i.e. 19.03)

Legal Description  The property listed in this publication is categorized as: (1) acreage, (2) improvements only, and (3) subdivided lots. Legal descriptions which describe acreage have several components. The first part describes where the parcel is located. Example: N1/2 SW1/4 SE1/4 means the North Half of the Southwest Quarter of the Southeast Quarter. The second part notes the section, township, and range, respectively, where the parcel is located. Example: 8 7N 16W means Section 8 of Township 7 North Range 16 West. The last part of the legal description is the parcel number or the number assigned to the property by the county assessor. If the parcel number is not available or does not exist a "N/A" is noted. The "code" which appears in this section refers to the page and line number where the entry is recorded in the Commissioner of State Lands office. The last number of the code refers to the original year of delinquency. Subdivided City Lots are also listed in this publication. The description of these lands is characterized by the lot and block number, the addition or subdivision, and city or town. The parcel and code number are the same as above. Improvements Only are improvements of buildings which have been assessed as real property, while the land on which they are located has been separately assessed for its value. They are usually described by their location on acreage or subdivided lots. The parcel and code number are the same as above. If a property address is listed in a parcel description, it is for informational purposes only. It is not a part of the legal description and, therefore, should not be relied on to determine the location of the parcel.